25-07-2026

192565089

RITHISH KUMAR D

CSE(CS)

ANNEXURE A

Solutions to Analytical Problem-Solving Questions

## Question 1: 2's Complement Addition and Subtraction (+45 and -23)

Step 1: Binary (2's Complement) Representation, 8-bit

+45  →  00101101 (magnitude bits of 45; sign bit = 0, so no conversion needed)

+23  →  00010111 (magnitude of 23)

-23  →  invert all bits of 00010111 → 11101000 , then add 1 → 11101001  (this is -23 in 2's complement)

Step 2: Addition — (+45) + (-23)

Since we are adding +45 and -23, we simply add their 2's-complement bit patterns and discard any carry out of the sign (MSB) position:

00101101   (+45)

+ 11101001   (-23)

------------

100010110   → discard the carry-out bit → 00010110

Result = 00010110 = +22 (decimal). This matches 45 - 23 = 22.

Step 3: Subtraction — (+45) - (-23)

Subtraction is performed by adding the 2's complement of the subtrahend. Since the subtrahend is already -23, its 2's complement is simply +23 (00010111):

00101101   (+45)

+ 00010111   (+23, i.e. 2's complement of -23)

------------

01000100

Result = 01000100 = +68 (decimal), which matches 45 - (-23) = 45 + 23 = 68.

Step 4: Overflow Detection

The processor detects overflow by comparing the carry into the sign bit (Cin) with the carry out of the sign bit (Cout). If they differ (Cin ≠ Cout), an overflow has occurred. Equivalently, overflow can only happen when both operands share the same sign but the result's sign differs from theirs.

- Addition (+45 and -23): operands have different signs, so overflow is impossible regardless of carry behaviour. Result +22 fits within the 8-bit signed range (-128 to +127) — valid.

- Subtraction, evaluated as (+45)+(+23): both operands are positive (same sign). Carry into sign bit = 0 and carry out of sign bit = 0, so Cin=Cout → no overflow. Result +68 is correctly represented since it lies within +127.

Interpretation of Result

Both operations produced correct, non-overflowed 8-bit signed results: 45 + (-23) = +22, and 45 - (-23) = +68. The processor's overflow flag remains 0 in both cases, confirming the results are valid representable signed integers.

## Question 2: 4-bit Carry Look-Ahead Adder (CLA) vs Ripple Carry Adder

Step 1: Generate and Propagate Signals

For each bit position i, two auxiliary signals are defined directly from the inputs (computed simultaneously, in parallel, for all bits):

- Generate:  Gi = Ai · Bi  — a carry is generated at this stage regardless of the incoming carry

- Propagate:  Pi = Ai ⊕ Bi  — an incoming carry would be passed through this stage

Step 2: Worked Example — A = 1011 (11), B = 0110 (6)

| Bit i | Ai | Bi | Gi = Ai·Bi | Pi = Ai⊕Bi |
| --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 |
| 2 | 0 | 1 | 0 | 1 |
| 3 | 1 | 0 | 0 | 1 |

Step 3: Look-Ahead Carry Equations (all carries computed in parallel from Gi, Pi, C0)

C1 = G0 + P0C0

C2 = G1 + P1G0 + P1P0C0

C3 = G2 + P2G1 + P2P1G0 + P2P1P0C0

C4 = G3 + P3G2 + P3P2G1 + P3P2P1G0 + P3P2P1P0C0

Substituting the values above (with C0 = 0):

C1 = 0 + 1·0 = 0   C2 = 1 + 0·0 = 1   C3 = 0 + 1·1 + 1·0·0 = 1   C4 = 0 + 1·0 + 1·1·1 = 1

Step 4: Sum Bits and Final Result

Each sum bit is Si = Pi ⊕ Ci:  S0 = 1⊕0 = 1,  S1 = 0⊕0 = 0,  S2 = 1⊕1 = 0,  S3 = 1⊕1 = 0.

Sum = S3S2S1S0 = 0001 with carry-out C4 = 1, giving the 5-bit result 10001 = 17, which correctly equals 11 + 6 = 17.

Step 5: Timing Comparison — CLA vs Ripple Carry Adder

- Ripple Carry Adder: each full adder must wait for the carry from the previous stage before it can produce its own sum and carry. For an n-bit adder the worst-case delay is proportional to n full-adder delays, i.e. O(n) gate delays in series (≈ 2n gate delays for n stages).

- Carry Look-Ahead Adder: all Gi and Pi signals are generated simultaneously (1 gate delay), and every carry Ci is computed directly from these signals through a two-level AND-OR network (2 more gate delays), independent of the other carries. Total delay is essentially constant, O(1)-ish (a small fixed number of gate delays), regardless of n.

Why CLA Improves Speed

Because a ripple carry adder's delay grows linearly with word length, it becomes a serious bottleneck in wide (32-bit, 64-bit) ALUs. The CLA removes this dependency chain by pre-computing every carry from Boolean expressions of the inputs, so all bit sums can be produced almost simultaneously.

## Question 3: Booth's Algorithm — Multiply -6 × +5

Step 1: Setup (5-bit 2's complement registers)

Multiplicand M = -6 = 11010   Multiplier Q = +5 = 00101

Accumulator A = 00000 (initial), Q-1 = 0 (initial). -M = +6 = 00110. The algorithm runs for 5 iterations (n = number of bits).

Booth's rule examines the pair (Q0, Q-1) at each step:

- 10 → subtract M from A  (A = A - M)

- 01 → add M to A  (A = A + M)

- 00 or 11 → no arithmetic operation

After the arithmetic step, the combined register (A, Q, Q-1) is shifted right arithmetically (sign bit of A preserved).

Step 2: Iteration Table

| Step | Q0,Q-1 | Operation | A after op | A,Q after arithmetic shift |
| --- | --- | --- | --- | --- |
| 1 | 1,0 | A = A - M | 00110 | A=00011 Q=00010 Q-1=1 |
| 2 | 0,1 | A = A + M | 11101 | A=11110 Q=10001 Q-1=0 |
| 3 | 1,0 | A = A - M | 00100 | A=00010 Q=01000 Q-1=1 |
| 4 | 0,1 | A = A + M | 11100 | A=11110 Q=00100 Q-1=0 |
| 5 | 0,0 | no operation | 11110 | A=11111 Q=00010 Q-1=— |

Step 3: Final Result

After 5 shifts, concatenating A and Q gives the 10-bit signed product:

A Q = 1111100010

Converting from 2's complement: this equals -30, which correctly matches -6 × 5 = -30.

Why Booth's Algorithm Is Efficient

Booth's algorithm handles signed numbers directly, without needing separate sign-magnitude conversion, because adding or subtracting the 2's-complement multiplicand naturally accounts for sign. By examining pairs of bits (Q0, Q-1), runs of consecutive 1s or 0s in the multiplier are skipped over using only shifts (00 or 11 patterns), so the number of actual add/subtract operations is reduced compared to a naive bit-by-bit multiplication scheme, especially for multipliers with long runs of identical bits. This reduces both the hardware operations and the overall multiplication time.

## Question 4: Division 13 ÷ 3 — Restoring vs Non-Restoring

Setup (4-bit registers)

Dividend Q = 1101 (13), Divisor M = 0011 (3), Accumulator A = 0000 initially. 4 iterations are performed (n = number of bits).

Part A: Restoring Division

At each step: shift (A,Q) left by 1, then A = A - M. If A becomes negative (MSB = 1), restore by adding M back to A and set Q0 = 0; otherwise leave A as is and set Q0 = 1.

| Step | After shift (A,Q) | A - M | Restore? | A,Q after step |
| --- | --- | --- | --- | --- |
| 1 | A=0001 Q=1010 | 1110 (neg) | Yes, restore | A=0001 Q=1010 |
| 2 | A=0011 Q=0100 | 0000 (pos) | No | A=0000 Q=0101 |
| 3 | A=0000 Q=1010 | 1101 (neg) | Yes, restore | A=0000 Q=1010 |
| 4 | A=0001 Q=0100 | 1110 (neg) | Yes, restore | A=0001 Q=0100 |

Final: Remainder A = 0001 = 1, Quotient Q = 0100 = 4. Check: 13 = 3×4 + 1 ✓

Part B: Non-Restoring Division

Rule: if the previous A ≥ 0, shift (A,Q) left then A = A - M; if the previous A < 0, shift (A,Q) left then A = A + M. After the operation, set Q0 = 1 if A ≥ 0, else Q0 = 0. No immediate restoring step is performed — a single correction is applied only at the very end if the final remainder is negative.

| Step | Previous A sign | Operation | A after op | Q0 |
| --- | --- | --- | --- | --- |
| 1 | ≥0 | shift, A = A - M | 1110 (neg) | 0 |
| 2 | <0 | shift, A = A + M | 0000 (pos) | 1 |
| 3 | ≥0 | shift, A = A - M | 1101 (neg) | 0 |
| 4 | <0 | shift, A = A + M | 1110 (neg) | 0 |

Final A is negative, so one corrective addition is applied: A = 1110 + 0011 = 0001. Result: Remainder = 0001 = 1, Quotient Q = 0100 = 4 — identical to the restoring method. Check: 13 = 3×4 + 1 ✓

Comparison and Justification

| Aspect | Restoring Division | Non-Restoring Division |
| --- | --- | --- |
| Correction step | Extra add-back (restore) performed every time A goes negative | No immediate correction; only one final correction if needed |
| Operations per iteration | Up to 2 (subtract, then possibly restore-add) | Exactly 1 (either add or subtract) |
| Speed / hardware complexity | Slower — restoring step adds extra cycles | Faster — consistent single operation per cycle, simpler control |

## Question 5: IEEE 754 Representation and Addition (-13.25)

Step 1: Convert -13.25 to Binary and Normalize

13.25 in binary: 13 = 1101, 0.25 = 0.01, so 13.25 = 1101.01. Normalizing to the form 1.xxxx × 2e:

1101.01 = 1.10101 × 2^3

Since the number is negative, the sign bit = 1.

Step 2: Single Precision (32-bit) Fields

Sign = 1;  Exponent (bias 127) = 3 + 127 = 130 = 10000010 (8 bits);  Mantissa (23 bits, fraction after the implicit leading 1) = 10101000000000000000000

| Sign (1 bit) | Exponent (8 bits) | Mantissa (23 bits) |
| --- | --- | --- |
| 1 | 10000010 | 10101000000000000000000 |

Step 3: Double Precision (64-bit) Fields

Sign = 1;  Exponent (bias 1023) = 3 + 1023 = 1026 = 10000000010 (11 bits);  Mantissa (52 bits) = 1010100000000000000000000000000000000000000000000

| Sign (1 bit) | Exponent (11 bits) | Mantissa (52 bits, truncated) |
| --- | --- | --- |
| 1 | 10000000010 | 1010100000...0 (52 bits) |

Step 4: Floating-Point Addition Example: (-13.25) + (+6.5)

Second operand: 6.5 = 110.1 = 1.101 × 2^2

- Align exponents: the smaller exponent (2) is raised to match the larger one (3) by shifting its mantissa right by 1 bit: 1.101 × 2^2 = 0.1101 × 2^3

- Operate on magnitudes: the operands have opposite signs, so the mantissas are subtracted: 1.10101 - 0.11010 = 0.11011 (result ×2^3)

- Normalize: shift left until the leading bit is 1 and adjust the exponent: 0.11011 × 2^3 = 1.1011 × 2^2

- Round: the mantissa already fits exactly within the available bits, so no rounding is required here.

- Assign sign: the operand with the larger magnitude (-13.25) determines the sign of the result, so the result is negative.

Final result: 1.1011 × 2^2 = 110.11 = -6.75, which correctly matches -13.25 + 6.5 = -6.75.

Interpretation and Notes on Precision

The exponent field encodes magnitude in powers of two while the mantissa encodes precision; aligning exponents before addition is essential so that bits of equal place value are combined.

Two practical issues arise in floating-point addition: (1) normalization is required whenever the raw result's leading bit is not 1, as shown above, and (2) rounding/precision loss occurs when the aligned mantissa has more bits than the format can store (23 bits for single precision, 52 for double), forcing the least-significant bits to be dropped or rounded — this is the source of the finite precision (and occasional representation error) inherent to IEEE 754 arithmetic.

RITHISH KUMAR.D(192565089)